# Low-Level Design (LLD)

Project Title: Multi-Tenant Data Center & WAN Infrastructure Engineering Specification
Design Phase: Low-Level Design (LLD)

1. Network Mapping Schemas & Mathematical Formulas

To automate configuration generation and prevent overlapping assignments, all VLAN, VNI, and VRF assignments are derived using deterministic formulas based on a unique Tenant ID (T), where $T \in [1, 100]$.

1.1 Structural Formulas

- Tenant Local Layer 2 LAN VLAN: 1000 + T

- Tenant Layer 3 WAN Transit/Edge VLAN: 2000 + T

- Layer 2 VNI (Bridging/Stretched LAN): 10000 + T

- Layer 3 VNI (Symmetric Routing Transit): 30000 + T

- Global Route Target (RT): 4200000000:30000+T

- Local Route Distinguisher (RD): <Local_Node_BGP_ASN>:30000+T

1.2 Multi-Tenant Control Plane Configuration Matrix

| Tenant ID (T) | VRF Name | L3 VNI (Symmetric) | L2 VNI (Stretched) | L3 Transit VLAN | L2 LAN VLAN | Route Distinguisher (RD)* | Route Target (RT) [RFC4364] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Tenant_1 | 30001 | 10001 | 2001 | 1001 | ASN:30001 | 4200000000:30001 |
| 2 | Tenant_2 | 30002 | 10002 | 2002 | 1002 | ASN:30002 | 4200000000:30002 |
| 3 | Tenant_3 | 30003 | 10003 | 2003 | 1003 | ASN:30003 | 4200000000:30003 |
| 4 | Tenant_4 | 30004 | 10004 | 2004 | 1004 | ASN:30004 | 4200000000:30004 |
| ... | Sequential | ... | ... | ... | ... | ... | ... |
| 50 | Tenant_50 | 30050 | 10050 | 2050 | 1050 | ASN:30050 | 4200000000:30050 |
| 51 | Tenant_51 | 30051 | 10051 | 2051 | 1051 | ASN:30051 | 4200000000:30051 |
| ... | Sequential | ... | ... | ... | ... | ... | ... |
| 100 | Tenant_100 | 30100 | 10100 | 2100 | 1100 | ASN:30100 | 4200000000:30100 |

*Engineering Note on RDs: In production configurations, ASN must be replaced by the device's specific local BGP instance number. For example, on the DC1 Edge Router (BGP ASN 4200000001), Tenant 1's local RD is configured as 4200000001:30001. On a DC1 Leaf Switch (BGP ASN 4200000101), the same tenant's local RD is 4200000101:30001.

2. Infrastructure Underlay & Physical IP Allocations

The underlay network strictly utilizes the 10.0.0.0/8 space. The following subnet allocations must be used when configuring interfaces, infrastructure loopbacks (VTEP/BGP peering), and point-to-point links.

| Subnet Block | Target Hardware / Interface Type | Engineering Implementation Details |
| --- | --- | --- |
| 10.1.0.0/16 | Data Center 1a Spine, Leaf, & Edge | Physical interface point-to-points, Loopback 0 (BGP), Loopback 1 (VTEP) |
| 10.2.0.0/16 | Data Center 1b Spine, Leaf, & Edge | Physical interface point-to-points, Loopback 0 (BGP), Loopback 1 (VTEP) |
| 10.3.0.0/16 | Data Center 2 Spine, Leaf, & Edge | Physical interface point-to-points, Loopback 0 (BGP), Loopback 1 (VTEP) |
| 10.4.0.0/16 | Data Center 3 Spine, Leaf, & Edge | Physical interface point-to-points, Loopback 0 (BGP), Loopback 1 (VTEP) |
| 10.10.0.0/16 | Core DCI Dark Fibre Routing | Inter-DC physical backbone links: 10.10.12.0/30 (DC1-DC2), 10.10.13.0/30 (DC1-DC3) |
| 10.100.0.0/14 | WAN Catalyst 8000 Edge Routers | External-facing physical interfaces connecting to WAN provider circuits |
| 10.200.0.0/14 | WAN Multi-Tenant GRE Tunnels | Point-to-point overlay tunnel networks (10.200.0.0/24 through 10.201.243.0/24) |

3. Data Center Tenant Compute Allocation Matrix

Tenant compute subnets are carved into dedicated, predictable /25 blocks per data center site from the 172.16.0.0/12 pool.

3.1 Distributed Anycast Gateway Behaviour

Every Data Center site uses an Anycast Gateway deployment pattern. The first usable IP address of each /25 subnet is configured identically across all local Leaf switches within that data center fabric. This allows virtual machines to move across leaf switches without changing their default gateway configuration.

[ Tenant 1 Compute Block: 172.16.0.0/23 ]

/               |               |               \

[ DC1 Fabric Pod ] [ DC2 Fabric Pod ] [ DC3 Fabric Pod ] [ DC4 Fabric Pod ]

172.16.0.0/25      172.16.0.128/25    172.16.1.0/25      172.16.1.128/25

Gateway: 172.16.0.1 Gateway: 172.16.0.129 Gateway: 172.16.1.1 Gateway: 172.16.1.129

3.2 Detailed Tenant IP Addressing Table (Tenants 1–100)

| Tenant ID | Global Block | DC1 Subnet & Anycast GW | DC2 Subnet & Anycast GW | DC3 Subnet & Anycast GW | DC4 Subnet & Anycast GW |
| --- | --- | --- | --- | --- | --- |
| Tenant 1 | 172.16.0.0/23 | Subnet: 172.16.0.0/25 Gateway: 172.16.0.1 | Subnet: 172.16.0.128/25 Gateway: 172.16.0.129 | Subnet: 172.16.1.0/25 Gateway: 172.16.1.1 | Subnet: 172.16.1.128/25 Gateway: 172.16.1.129 |
| Tenant 2 | 172.16.2.0/23 | Subnet: 172.16.2.0/25 Gateway: 172.16.2.1 | Subnet: 172.16.2.128/25 Gateway: 172.16.2.129 | Subnet: 172.16.3.0/25 Gateway: 172.16.3.1 | Subnet: 172.16.3.128/25 Gateway: 172.16.3.129 |
| Tenant 3 | 172.16.4.0/23 | Subnet: 172.16.4.0/25 Gateway: 172.16.4.1 | Subnet: 172.16.4.128/25 Gateway: 172.16.4.129 | Subnet: 172.16.5.0/25 Gateway: 172.16.5.1 | Subnet: 172.16.5.128/25 Gateway: 172.16.5.129 |
| Tenant 4 | 172.16.6.0/23 | Subnet: 172.16.6.0/25 Gateway: 172.16.6.1 | Subnet: 172.16.6.128/25 Gateway: 172.16.6.129 | Subnet: 172.16.7.0/25 Gateway: 172.16.7.1 | Subnet: 172.16.7.128/25 Gateway: 172.16.7.129 |
| ... | Sequential Gap | ... | ... | ... | ... |
| Tenant 50 | 172.16.98.0/23 | Subnet: 172.16.98.0/25 Gateway: 172.16.98.1 | Subnet: 172.16.98.128/25 Gateway: 172.16.98.129 | Subnet: 172.16.99.0/25 Gateway: 172.16.99.1 | Subnet: 172.16.99.128/25 Gateway: 172.16.99.129 |
| Tenant 51 | 172.16.100.0/23 | Subnet: 172.16.100.0/25 Gateway: 172.16.100.1 | Subnet: 172.16.100.128/25 Gateway: 172.16.100.129 | Subnet: 172.16.101.0/25 Gateway: 172.16.101.1 | Subnet: 172.16.101.128/25 Gateway: 172.16.101.129 |
| ... | Sequential Gap | ... | ... | ... | ... |
| Tenant 100 | 172.16.198.0/23 | Subnet: 172.16.198.0/25 Gateway: 172.16.198.1 | Subnet: 172.16.198.128/25 Gateway: 172.16.198.129 | Subnet: 172.16.199.0/25 Gateway: 172.16.199.1 | Subnet: 172.16.199.128/25 Gateway: 172.16.199.129 |

4. Remote Node Branch Site Allocations

Remote site networks are allocated sequentially from the 10.100.0.0/12 pool using fixed /22 blocks.

Example Site Layout for Remote Node X (/22 split):

- Local User Data Subnet:  First /24 block

- Voice / VoIP Subnet:     Second /24 block

- Local Mgmt & Loopbacks:  Fourth /24 block

| Remote Node ID | Configured CIDR Block | Usable Host IP Address Range | Application Description |
| --- | --- | --- | --- |
| Remote Node 1 | 10.100.0.0/22 | 10.100.0.1 – 10.100.3.254 | Branch user LANs, local peripheral devices, and edge router interfaces. |
| Remote Node 2 | 10.100.4.0/22 | 10.100.4.1 – 10.100.7.254 | Branch user LANs, local peripheral devices, and edge router interfaces. |
| Remote Node 3 | 10.100.8.0/22 | 10.100.8.1 – 10.100.11.254 | Branch user LANs, local peripheral devices, and edge router interfaces. |
| ... | Sequential /22 Blocks | ... | ... |
| Remote Node 500 | 10.107.208.0/22 | 10.107.208.1 – 10.107.211.254 | Branch user LANs, local peripheral devices, and edge router interfaces. |

5. WAN GRE Tunnel Overlay (DMVPN)

Because DMVPN uses a shared multipoint subnet per cloud, we no longer use /31 subnets for every link. Instead, each of the 8 DMVPN clouds gets its own large /22 subnet (1,024 IP addresses).  Within each /22 DMVPN cloud:

a) IPs .1 to .4 are statically assigned to the 4 Data Center Hub Edges.

b) IPs .5 to .504 are dynamically or statically assigned to the 500 remote spokes.

This approach organizes the 8 QoS DMVPN tunnels into an exceptionally neat, predictable slice of your 10.200.0.0/14 block:

| Tunnel ID | QOS / Traffic Class | DSCP / PHB | DMVPN Network Block | Hub Edge IPs (DC1 – DC4) | Spoke Nodes IP Range |
| --- | --- | --- | --- | --- | --- |
| Tunnel 101 | Voice / Real-Time | EF | 10.200.0.0/22 | 10.200.0.1 – 10.200.0.4 | 10.200.0.5 – 10.200.1.248 |
| Tunnel 102 | Video Conferencing | AF41 | 10.200.4.0/22 | 10.200.4.1 – 10.200.4.4 | 10.200.4.5 – 10.200.5.248 |
| Tunnel 103 | Critical Apps (Data 1) | AF31 | 10.200.8.0/22 | 10.200.8.1 – 10.200.8.4 | 10.200.8.5 – 10.200.8.248 |
| Tunnel 104 | Business Apps (Data 2) | AF21 | 10.200.12.0/22 | 10.200.12.1 – 10.200.12.4 | 10.200.12.5 – 10.200.12.248 |
| Tunnel 105 | Network Operations | CS6 | 10.200.16.0/22 | 10.200.16.1 – 10.200.16.4 | 10.200.16.5 – 10.200.16.248 |
| Tunnel 106 | Scavenger Traffic | CS1 | 10.200.20.0/22 | 10.200.20.1 – 10.200.20.4 | 10.200.20.5 – 10.200.20.248 |
| Tunnel 107 | Out-of-Band Mgmt | CS2 | 10.200.24.0/22 | 10.200.24.1 – 10.200.24.4 | 10.200.24.5 – 10.200.24.248 |
| Tunnel 108 | Best Effort Default | BE | 10.200.28.0/222 | 10.200.28.1 – 10.200.28.4 | 10.200.24.5 – 10.200.24.248 |

*It should be noted: These 8 core QoS clouds take up a tiny portion of your allocation—specifically 10.200.0.0 through 10.200.31.255—leaving massive blocks free in the rest of the /14 space for future expansion.

6. Infrastructure Underlay & Overlay ASN Matrix

To avoid conflicts with public internet routing, the entire infrastructure strictly utilizes 4-byte private ASN space (4200000000 to 4294967294).

The allocation block is structured mathematically to match your location footprints:

- Data Center Core Blocks: 4200001XXX to 4200004XXX

- WAN Edge & Transport: 421000000X

- Remote Branch Nodes: 4220000001 to 4220000500 (Mapping 1:1 with Remote Node ID)

6.1 Data Center ASN Matrix

| Location | Node Type | Physical Quantity | BGP Peer Type | Assigned ASN / ASN Range | Engineering Rules |
| --- | --- | --- | --- | --- | --- |
| DC1a | Spine Switches | 2-4 | Underlay (eBGP) | 4200001000 | Common ASN shared across all spines. |
| DC1a | Leaf Switches | Up to 64 | Underlay (eBGP) | 4200001000 – 4200001064 | Unique ASN per Leaf pair / switch to prevent routing loops in underlay. |
| DC1a | EVPN Route Reflector | 2 | Overlay (iBGP) | 4200001100 | Peered via Loopback 0. Reflects EVPN routes to all local VTEPS. |
| DC1b | Spine Switches | 2-4 | Underlay (eBGP) | 4200001500 | Common ASN shared across all spines. |
| DC1b | Leaf Switches | Up to 64 | Underlay (eBGP) | 42000015001 – 4200001564 | Unique ASN per Leaf pair / switch to prevent routing loops in underlay. |
| DC1b | EVPN Route Reflector | 2 | Overlay (iBGP) | 4200001600 | Peered via Loopback. Reflects EVPN routes to all local VTEPS. |
| DC2 (DR Hub) | Spine Switches | 2-4 | Underlay (eBGP) | 4200002000 | Common ASN shared across all spines. |
| DC2 (DR Hub) | Leaf Switches | Up to 64 | Underlay (eBGP) | 4200002000– 4200002064 | Unique ASN per Leaf pair / switch to prevent routing loops in underlay. |
| DC2 (DR Hub) | EVPN Route Reflector | 2 | Overlay (iBGP) | 4200002100 | Peered via Loopback 0. Reflects EVPN routes to all local VTEPS. |
| DC3 (DR Hub) | Spine Switches | 2-4 | Underlay (eBGP) | 4200003000 | Common ASN shared across all spines. |
| DC3 (DR Hub) | Leaf Switches | Up to 64 | Underlay (eBGP) | 4200003000– 4200003064 | Unique ASN per Leaf pair / switch to prevent routing loops in underlay. |
| DC3 (DR Hub) | EVPN Route Reflector | 2 | Overlay (iBGP) | 4200003100 | Peered via Loopback 0. Reflects EVPN routes to all local VTEPS. |
| DC4 (DR Hub) | Spine Switches | 2-4 | Underlay (eBGP) | 4200004000 | Common ASN shared across all spines. |
| DC4 (DR Hub) | Leaf Switches | Up to 64 | Underlay (eBGP) | 4200004000– 4200004064 | Unique ASN per Leaf pair / switch to prevent routing loops in underlay. |
| DC4 (DR Hub) | EVPN Route Reflector | 2 | Overlay (iBGP) | 4200004100 | Peered via Loopback 0. Reflects EVPN routes to all local VTEPS. |

6.2 WAN Edge ASN Matrix

| Network Zone | Device Site | BGP Role | Peer Type | Assigned ASN | Alignment with previous schema maps |
| --- | --- | --- | --- | --- | --- |
| DC WAN Edge | DC 1 | Multi-Tenant WAN hub | eBGP | 4210000001 | Peers with DC Leaf switches on L3 Transit VLANs |
| DC WAN Edge | DC 2 | Multi-Tenant WAN hub | eBGP | 4210000002 | Hub peer for DR traffic paths |
| DC WAN Edge | DC 3 | Multi-Tenant WAN hub | eBGP | 4210000003 | Hub peer for DR traffic paths |
| DC WAN Edge | DC 3 | Multi-Tenant WAN hub | eBGP | 4210000004 | Hub peer for DR traffic paths |

6.2 Remote Node ASN Matrix

| Network Zone | Device Site | BGP Role | Peer Type | Assigned ASN | Alignment with previous schema maps |
| --- | --- | --- | --- | --- | --- |
| Branch Edge | Remote Node 1 | Tenant Spoke Router | eBGP | 4220000001 | Hosts customer IP Block – 10.100.0.0/22. |
| Branch Edge | Remote Node 2 | Tenant Spoke Router | eBGP | 4220000002 | Hosts customer IP Block – 10.100.4.0/22. |
| Branch Edge | Remote Node 3 | Tenant Spoke Router | eBGP | 4220000003 | Hosts customer IP Block – 10.100.8.0/22. |
| Branch Edge | Remote Node 4 | Tenant Spoke Router | eBGP | 4220000004 | Hosts customer IP Block – 10.100.12.0/22. |
| … | Sequential Sites | Tenant Spoke Router | eBGP | Sequential | Maps 1:1 with remote node id. |
| Branch Edge | Remote Node 500 | Tenant Spoke Router | eBGP | 4220000500 | Hosts customer IP Block – 10.107.208.0/22. |

6.3 Multi-Tenant Route Distinguisher (RD) Alignment

As established in the core schema mapping formula, Route Distinguishers use the local node's physical BGP instance prefix combined with the L3 VNI (ASN:30000+T).

With a concrete ASN mapping in place, the production engineering configurations for Tenant 1 (T=1, L3 VNI = 30001) will compile as follows:

- DC1a Leaf 1 (Physical ASN 4200001001): 4200001001:30001

- DC1b Leaf 1 (Physical ASN 4200001501): 4200001501:30001

- DC2 Leaf 1 (Physical ASN 4200002001): 4200002001:30001

- DC3 Leaf 1 (Physical ASN 4200003001): 4200003001:30001

- DC4 Leaf 1 (Physical ASN 4200004001): 4200004001:30001

- DC1 WAN Edge (Physical ASN 4210000001): 4210000001:30001

7 Multi-Tenant Out-of-Band (OOB) Management Architecture

7.1 Architectural Strategy

- Physical Isolation: Tenant management ports cable into a dedicated physical infrastructure (OOB Management Switches) completely separate from the production Spine/Leaf/Edge fabric.

- Control-Plane Isolation: The physical OOB switches run a dedicated management VRF per tenant (Mgmt_Tenant_T).

- Secure Transit to Remote Sites: Tenant management traffic is backhauled from the data center to the tenant's remote branch nodes via the existing WAN Edge infrastructure, utilizing dedicated management GRE tunnels mapped to the tenant's VRF.

7.2 OOB Schema Mapping

To ensure perfect alignment with our 5-digit mathematical formula, we extend the LLD schema matrix to include dedicated management VLANs, VRFs, and IP blocks.

7.2.1 Extended Mapping

- Tenant Local OOB Management VLAN: 9000 + T (Configured on the physical OOB switches).

- Tenant OOB Management VRF Name: Mgmt_Tenant_T

- Tenant WAN OOB Transit VLAN: 2900 + T (The infrastructure transit VLAN between the OOB Core and the Catalyst 8K WAN Edge).

7.3 OOB IP Allocation Strategy

To keep the primary tenant compute pool (172.16.0.0/12) entirely clean for production applications, A dedicated block from the 172.16.0.0/12 pool will be used to prevent a distinct private block to prevent overlap.

The 172.31.0.0/16 block has been designated strictly for Multi-Tenant OOB Management across all four data centers. This provides each tenant with a contiguous /24 block (256 IPs) for infrastructure management, split into 4 equal /26 subnets (64 IPs per data center):

7.3.1 Multi-Tenant OOB Schema Matrix

| Mgmt VRF Name | OOB VLAN | WAN VLAN | DC1 OOB Subnet & Gateway | DC2 OOB Subnet & Gateway | Remote Node Mgmt Allocation |
| --- | --- | --- | --- | --- | --- |
| Mgmt_Tenant_1 | 9001 | 2901 | Subnet: 172.31.0.0/26 Gateway: 172.31.0.1 | Subnet: 172.31.0.64/26 Gateway: 172.31.0.65 | 10.100.3.0/24 |
| Mgmt_Tenant_2 | 9002 | 2902 | Subnet: 172.31.1.0/26 Gateway: 172.31.1.1 | Subnet: 172.31.1.64/26 Gateway: 172.31.1.65 | 10.100.7.0/24 |
| Mgmt_Tenant_3 | 9003 | 2903 | Subnet: 172.31.2.0/26 Gateway: 172.31.0.1 | Subnet: 172.31.2.64/26 Gateway: 172.31.0.65 | 10.100.11.0/24 |
| Mgmt_Tenant_4 | 9004 | 2904 | Subnet: 172.31.3.0/26 Gateway: 172.31.3.1 | Subnet: 172.31.3.64/26 Gateway: 172.31.3.65 | 10.100.15.0/24 |
| Sequential Gap | ... | ... | ... | ... | ... |
| Mgmt_Tenant_100 | 9100 | 2100 | Subnet: 172.31.99.0/26 Gateway: 172.31.99.1 | Subnet: 172.31.99.64/26 Gateway: 172.31.99.65 | 10.107.211.0/24 |