# High-Level Design (HLD)

Project Title: Multi-Tenant Data Center & WAN Infrastructure Architecture
Design Phase: High-Level Design (HLD)

# 1. Executive Summary & Design Principles

This document outlines the architectural blueprint for a highly scalable, multi-tenant network infrastructure spanning multiple Data Centers (DCs) and 500 remote sites. The design natively accommodates a growth path from 1 tenant (Day 1) to 100 tenants (Day 3), supporting modern EVPN-VXLAN encapsulation alongside legacy VRF-Lite deployments.

## Key Design Pillars

- Strict Multi-Tenancy: Complete control-plane and data-plane isolation using Virtual Routing and Forwarding (VRFs), Multiprotocol BGP (MP-BGP), and VXLAN Network Identifiers (VNIs).

- Separation of Layers: Underlay routing (physical reachability) is entirely decoupled from Overlay routing (tenant traffic).

- Deterministic Traffic Flow: DC1 acts as the Primary hub, while DC2 and DC3 operate as Active/Standby Disaster Recovery (DR) hubs.

- Vendor Agnostic Compute Edge: Open-standards EVPN-VXLAN and eBGP allow seamless integration of heterogeneous hardware (e.g., Cisco and Dell leaf/compute switches).

# 2. Multi-Tenant Logical Architecture

The architecture enforces a strict 1-to-1-to-1 relationship between a Tenant, their Layer 3 VRF, and their corresponding Virtual Networks across the data center fabrics.

+-----------------------------------+

|        Tenant Virtual Layer       |

|  [VRF] [Symmetric L3 VNI Route]   |

+-----------------------------------+

|

+------------------------------+------------------------------+

|                                                             |

+-----------------------------------+                 +-----------------------------------+

|      Data Center Overlay          |                 |        WAN Edge Overlay           |

| [L2 VNI Stretched/Local Bridging] |                 | [VRF-Lite / GRE Multi-Tenant Tunnels]|

+-----------------------------------+                 +-----------------------------------+

# 2.1 Control Plane & Data Plane Separation

- Underlay Network: Handles physical connectivity, infrastructure loopbacks, and point-to-point links. It uses high-performance Interior Gateway Protocols (IGP) or External BGP (eBGP) to establish IP reachability between all physical switch interfaces (VTEPs). Tenant traffic never runs natively within this layer.

- Overlay Network: Uses EVPN as the control plane to distribute MAC and IP reachability via MP-BGP. VXLAN encapsulation is used in the data plane to tunnel tenant traffic securely over the underlay.

# 2.2 Route Target & Distinguisher Strategy

To support scalability and multi-site multi-tenancy, Route Targets (RT) and Route Distinguishers (RD) are assigned based on [RFC4364]:

- Route Target (RT): Kept globally identical across all data center fabrics and WAN nodes for a given tenant to allow seamless EVPN route importing and exporting.

- Route Distinguisher (RD): Must be locally unique on every switch and router across the 4 data centers to maintain paths from different VTEPs. The RD prefix matches the local physical switch/router BGP Autonomous System Number (ASN) or Loopback, ensuring uniqueness.

# 3. Global IP Addressing & Summarisation Strategy

To ensure clean BGP summarisation and prevent routing table bloat across the 500 remote nodes, the global network infrastructure utilizes strict, non-overlapping block allocations from private IP spaces (10.0.0.0/8 and 172.16.0.0/12).

[ Global Private IP Space ]

/           \

/             \

[ Infrastructure / Underlay / WAN ]         [ Tenant Overlay Compute ]

10.0.0.0/8                              172.16.0.0/12

# 3.1 Infrastructure Underlay Allocation Strategy

The infrastructure blocks are divided to isolate data center fabrics, inter-DC connectivity, and WAN provider networks:

- DC Fabric Underlays: Dedicated /16 blocks per data center footprint to isolate local Leaf-Spine physical networks.

- Data Center Interconnect (DCI): Dedicated transit blocks for physical Dark Fibre links connecting the core sites.

- WAN Provider Transport: Separate blocks for the outer physical interfaces of the edge routers connected to service provider circuits.

- WAN GRE Tunnel Overlay: Point-to-point transit networks used to build the multi-tenant WAN fabric.

# 3.2 Tenant Overlay Compute Pool

The entire 172.16.0.0/16 range (65,536 addresses) is dedicated strictly to multi-tenant compute environments (virtual machines, bare-metal application servers, and databases) residing within the data center fabrics.

- Sizing Rules: Every tenant is allocated a contiguous /23 block (512 IP addresses). A single /16 block yields exactly 128 unique tenant blocks, which fits the Day 3 target of 100 tenants with a 28-block growth buffer.

- Fabric Splitting: To maintain clean summary boundaries and predictable symmetric routing, each tenant's /23 block is divided into four equal /25 subnets—one per data center site.

# 3.3 Remote Edge Nodes (Non-Compute Branches)

Branch offices and remote retail/corporate sites use a distinct, massive block allocated from the 10.0.0.0/8 space to keep them architecturally separate from the data center underlay and compute blocks.

- Global Nodes Pool: 10.100.0.0/12 (Spanning 10.100.0.0 to 10.115.255.255), offering 16 unique /16 blocks.

- Branch Capacity: Each of the 500 remote sites is allocated a large /22 block (1,024 IP addresses) to accommodate local client LANs, guest access, voice networks, and branch management. This design supports up to 4,096 unique sites, meaning the Day 3 footprint of 500 branches consumes only ~12% of the allocated pool, leaving room for future expansion.